COSMOS COLLEGE OF MANAGEMENT & TECHNOLOGY

(Affiliated to Pokhara University)

Sitapaila, Kathmandu, Nepal

AN ENGINEERING PROJECT REPORT

ON

Digital Clock (MM:SS) Using ATmega32 Microcontroller

Submitted to:

Department of  ICT

Bachelor of Engineering in Computer Engineering

Submitted By:

Oshan Bajracharya

Roll No: 230328

Date of Submission:
1th FEB  2026

# INTRODUCTION

This project focuses on the design and implementation of a digital clock using the ATmega32 microcontroller. The clock displays time in Minutes and Seconds (MM:SS) format using four common cathode 7-segment displays. Embedded systems combine hardware and software to perform specific tasks efficiently. This project demonstrates important embedded system concepts such as port interfacing, multiplexing, and software-based timing.

# OBJECTIVE

• To interface multiple 7-segment displays with a microcontroller
• To understand and implement display multiplexing
• To generate accurate time delays using software
• To design a real-time digital clock system
• To gain practical knowledge of embedded C programming

# COMPONENTS USED

# Atmega32 microcontroller

# 4 × common cathode 7-segment displays

# 5V power supply

# Connecting wires

# 8 MHz system clock (internal or external)

# CIRCUIT DESCRIPTION

- PortC (PC0–PC7) of the ATmega32 is directly connected to the segment pins (a–g, dp) of all four 7-segment displays.

- PortD pins PD0–PD3 are used as digit select (common cathode) lines.

- Each display is enabled one at a time by controlling PD0–PD3 while segment data is sent through PortC.

- No current-limiting resistors are present in the shown circuit; segments are directly driven from the microcontroller port pins.

# WORKING PRINCIPLE

The system uses a lookup table to generate segment patterns for digits 0–9. Multiplexing enables one display at a time with a small delay so that all digits appear continuously ON. The program increments seconds every one-second delay. When seconds reach 60, minutes increment. After 59:59, the clock resets to 00:00.

# SOFTWARE DESIGN

The program initializes PORTC and PORTD as outputs. A display function handles digit scanning and refreshing. The main loop splits minutes and seconds into individual digits and displays them continuously. Delay functions create approximate 1-second timing.

ADVANTAGES

• Efficient use of I/O pins
• Low cost and simple hardware
• Good learning model for real-time embedded systems

# LIMITATIONS

• Timing accuracy depends on software delay
• No hour display
• No real-time clock module

# FUTURE ENHANCEMENTS

• Add RTC module (DS3231)
• Add hour display (HH:MM:SS)
• Add buttons for time setting
• Use timer interrupts instead of delays

# CONCLUSION

The digital clock project successfully demonstrates embedded system fundamentals including hardware interfacing, multiplexing, and timing control using the ATmega32 microcontroller. The system accurately displays time in MM:SS format and serves as a strong foundation for more advanced real-time systems.